TCP/IP Troubleshooting Lab

Diagnosing Network Faults on the 172.16.100.x/24 Subnet

Screen-Reader Accessible  |  Non-Graded Practice Exercise

# Accessibility Notes

SCREEN READER TIP: Commands are on their own line in a blue-bordered block. Use review keys to read each character before pressing Enter. Do not press Enter until you have heard the full command.

SCREEN READER TIP: Sample output blocks have a red left border. They represent what your screen reader would announce after running a command. They are not commands — do not type them.

SCREEN READER TIP: Headings are Heading 1, 2, and 3. In NVDA or JAWS browse mode, press H to jump between headings, T to jump between tables, and arrow keys to navigate table cells.

# Lab Overview

In this exercise you and your partner act as network technicians responding to fault reports on the isolated 172.16.100.x/24 subnet. One of you runs the diagnostic commands while the other acts as the remote host. You swap roles between scenarios.

This lab is not graded. The goal is to practice using each command until it feels natural and you understand what its output is telling you. Each scenario presents a symptom, guides you to the right command, and asks you to apply a fix.

## Your Addresses for This Lab

- Student B  PC1:   172.16.100.10   Mask: 255.255.255.0   Gateway: 172.16.100.1

- Student B  PC2:   172.16.100.11   Mask: 255.255.255.0   Gateway: 172.16.100.1

- TP-Link LAN Gateway:   172.16.100.1

NOTE: The DNS server configured on both machines is 8.8.8.8 (Google Public DNS). This server can resolve public internet hostnames but it has no knowledge of local lab names such as pc1-student-b or pc2-student-b. Local name resolution in this lab is done through the hosts file, explained in Scenario 2.

# Command Reference

The table below has 14 data rows and 3 columns: Command, What It Shows, and When to Use It. Review this before starting the scenarios.

| Command | What It Shows | When to Use It |
| --- | --- | --- |
| ipconfig | IP address, subnet mask, and default gateway for each adapter | Quick sanity-check that an address is assigned and correct |
| ipconfig /all | Full detail: MAC address, DHCP enabled/disabled, DNS server address, lease times | Deep-dive when basic config looks wrong or DHCP is suspect |
| ipconfig /release | Releases the current DHCP lease without requesting a new one | Pair with /renew to force a fresh DORA cycle |
| ipconfig /renew | Drops current DHCP lease and requests a new one from the DHCP server | When DHCP address is missing, stale, or shows 169.254.x.x |
| ipconfig /displaydns | Shows the local DNS resolver cache (recently looked-up names and their IPs) | Check whether a cached entry is stale or incorrect |
| ipconfig /flushdns | Clears the DNS resolver cache entirely | Force a fresh DNS lookup after editing the hosts file |
| nslookup <hostname> | Queries the configured DNS server and returns what it knows about the hostname | Test whether 8.8.8.8 can resolve a public name |
| nslookup <hostname> <server> | Queries a specific DNS server instead of the default | Compare results between 8.8.8.8 and a local source |
| netstat -an | All active TCP/UDP connections and listening ports with their states | Check if a service is running and whether a session is established |
| netstat -n | Active connections with numeric IPs only (no DNS reverse lookup) | Faster output when you only need IP addresses and ports |
| arp -a | ARP cache: all known IP-to-MAC address mappings on the local subnet | Verify Layer 2 reachability; detect IP address conflicts |
| arp -d * | Clears the entire ARP cache | Force fresh ARP resolution after an address change or conflict fix |
| ping <IP> | Tests basic ICMP reachability to a host | Always the first test in any troubleshooting sequence |

# Troubleshooting Method: Work Bottom-Up

Always start at the lowest layer and work upward. Fixing a higher-layer problem while a lower-layer problem exists is wasted effort.

Step 1  Physical:           Is the cable plugged in? Is the adapter enabled in Device Manager?

Step 2  IP Configuration:   Does the machine have the correct IP, mask, and gateway?

Commands:           ipconfig   and   ipconfig /all

Step 3  Reachability:       Can packets travel between the two machines?

Command:            ping 172.16.100.11

Step 4  ARP:               Does the machine know the MAC address of the target?

Command:           arp -a

Step 5  Name Resolution:   Can the hostname be translated to an IP address?

Commands:          ipconfig /displaydns   and   ipconfig /flushdns   and   nslookup (for public names)

Important:         8.8.8.8 will NOT resolve local lab hostnames. Use the hosts file for local names.

Step 6  Transport Service: Is the destination service actually listening on the expected port?

Command:           netstat -an

# Quick Symptom-to-Command Map

This table has 6 data rows and 3 columns: Symptom, First Command to Run, and What to Look For.

| Symptom | First Command to Run | What to Look For |
| --- | --- | --- |
| Cannot reach any host on 172.16.100.x | ipconfig | Missing or wrong IP address or subnet mask |
| IP address is 169.254.x.x (APIPA) | ipconfig /release then ipconfig /renew | DHCP server not responding; verify server is online |
| Local hostname not resolving (expected — 8.8.8.8 only resolves public names) | ipconfig /displaydns then check hosts file | Entry absent from cache; add hostname to hosts file manually |
| Public hostname resolves to wrong IP | ipconfig /displaydns | Stale cache entry; run ipconfig /flushdns |
| Service not connecting on port 5000 | netstat -an | Port 5000 absent from LISTENING column on server machine |
| IP conflict suspected (intermittent ping) | arp -a | Same IP appears with two different MAC addresses |

# Warm-Up: Capture a Healthy Baseline

Before any fault scenario, run the sequence below to record what a working network looks like. If you spot a problem during the warm-up, tell your instructor before proceeding.

## Step 1 — Quick IP check

ipconfig

Expected: IP 172.16.100.10, Mask 255.255.255.0, Gateway 172.16.100.1. If any value is wrong, stop and fix it before continuing.

## Step 2 — Full detail check

ipconfig /all

Listen for: Physical Address (your MAC address), DHCP Enabled (Yes or No), and DNS Servers. The DNS server should show 8.8.8.8. Note all three values for comparison later.

## Step 3 — Reachability test

ping 172.16.100.11

Expected: four replies with 0 percent packet loss. If ping fails, troubleshoot IP configuration before any other step.

## Step 4 — ARP cache check

arp -a

After a successful ping, your partner's IP 172.16.100.11 should appear with their MAC address. If the same IP shows two different MACs, there is an IP conflict. Report it to your instructor immediately.

## Step 5 — DNS cache and hosts file check

ipconfig /displaydns

The cache will likely be empty at the start of the lab. That is normal with 8.8.8.8 as the DNS server, because no public lookups have been made yet. Local lab hostnames are not in the cache because 8.8.8.8 does not know them.

To verify local name resolution, check the hosts file. On Windows the hosts file is located at:

C:\Windows\System32\drivers\etc\hosts

Open it with Notepad run as Administrator. Each line maps a hostname to an IP address. A correct entry for your lab looks like:

172.16.100.11    pc2-student-b

If this line is absent or incorrect, local hostname resolution will fail even though 8.8.8.8 is working perfectly.

## Step 6 — nslookup test (public name only)

nslookup google.com

This tests whether 8.8.8.8 is reachable and responding. A successful result returns an IP address for google.com. If this fails, there is a connectivity or DNS server problem outside your subnet.

NOTE: Do NOT use nslookup to look up local lab hostnames such as pc2-student-b. Because 8.8.8.8 is your DNS server, it will always return a failure for local names. Use the hosts file instead.

## Step 7 — Active connections check

netstat -an

At baseline there should be no ESTABLISHED sessions unless other software is running. Note any ports already in LISTENING state.

# Troubleshooting Scenarios

Work through each scenario in order. Read the fault report, choose the right command, run it, interpret the output, and apply the fix. Discuss every step with your partner before moving on.

Scenario 1: The Mystery Address

Fault report: Your machine cannot reach 172.16.100.11 even though the cable is plugged in. A classmate changed some settings and you do not know what they changed.

### Your Task

Use ipconfig to diagnose the problem from the command line only. Do not open any GUI network settings panels.

- Run ipconfig and listen carefully to every value.

ipconfig

Sample of a broken output your screen reader might announce:

IPv4 Address . . . . : 192.168.1.50

Subnet Mask  . . . . : 255.255.255.0

Default Gateway  . . : 192.168.1.1

The address 192.168.1.50 is on a completely different network. Your machine cannot reach anything on 172.16.100.x until this is corrected.

- Run ipconfig /all to determine whether this is a static address or a DHCP mistake.

ipconfig /all

Listen for DHCP Enabled. If it says No, someone manually entered the wrong address. If it says Yes, the DHCP server handed out an address from the wrong pool.

- Fix the address.

If static: open network adapter properties and set IP to 172.16.100.10, mask to 255.255.255.0, gateway to 172.16.100.1.

If DHCP: force a fresh lease:

ipconfig /release

ipconfig /renew

- Verify the fix.

ipconfig

Confirm you hear 172.16.100.10, mask 255.255.255.0, gateway 172.16.100.1.

- Confirm reachability to your partner.

ping 172.16.100.11

### Discussion Questions — Scenario 1

- How did ipconfig alone reveal the problem before you even tried to ping?

- What is the difference between a wrong IP address and a missing IP address?

- Why can a machine on 192.168.1.x never reach 172.16.100.x without a router?

Scenario 2: The Hostname That Goes Nowhere

Fault report: You can ping 172.16.100.11 by IP address but when you try to connect using the hostname pc2-student-b, nothing works. Your partner has a service running on port 8080.

### Background: Why nslookup Will Not Help Here

Your DNS server is 8.8.8.8. This is a public DNS server run by Google. It knows the addresses of websites like google.com and bbc.co.uk, but it has no knowledge of private local names like pc2-student-b. Running nslookup pc2-student-b against 8.8.8.8 will always fail, not because anything is broken, but because it is simply not the right tool for local names.

Local name resolution on this subnet uses the hosts file. The hosts file is a plain text file on your machine that maps hostnames to IP addresses before DNS is even consulted. If a hostname appears in the hosts file, Windows uses that entry directly without asking 8.8.8.8 at all.

### Your Task

Since ping works, Layer 3 is fine. The fault is in local name resolution. Investigate the hosts file and the DNS cache, then verify the service is running.

- Check the DNS cache first to see if any entry exists for pc2-student-b.

ipconfig /displaydns

Scroll through the output and listen for any entry named pc2-student-b. If nothing is there, Windows has no cached record for this name and will not be able to resolve it.

- Open the hosts file to check whether a manual entry exists.

Open Notepad as Administrator, then open the file at this path:

C:\Windows\System32\drivers\etc\hosts

Listen through each line. A correct entry sounds like:

172.16.100.11    pc2-student-b

If this line is missing or points to the wrong IP, that is the fault. Add or correct the line, then save the file.

- After editing the hosts file, flush the DNS cache so Windows picks up the new entry immediately.

ipconfig /flushdns

- Verify the hosts file entry is now in the DNS cache.

ipconfig /displaydns

You should now hear an entry for pc2-student-b showing 172.16.100.11.

- Use nslookup to confirm the entry resolves correctly. Because the hosts file is checked before DNS, nslookup will return the hosts file value rather than querying 8.8.8.8.

nslookup pc2-student-b

Expected output:

Server:   8.8.8.8

Name:     pc2-student-b

Address:  172.16.100.11

If you still see a failure here, the hosts file entry was not saved correctly. Open it again and verify the line is present and has no leading spaces or extra characters.

- Now check whether the service on port 8080 is actually running on your partner's machine. Ask your partner to run:

netstat -an

Your partner listens for port 8080 in LISTENING state:

TCP    0.0.0.0:8080    0.0.0.0:0    LISTENING

If port 8080 is absent, the service is not running. Correct name resolution will not help if the service itself is down.

- If the service is not running, your partner starts a listener to simulate it:

nc -l -p 8080

- Your partner re-checks netstat to confirm the port is now listening.

netstat -an

- You connect from your machine using the hostname (not the IP) to confirm end-to-end resolution and connectivity.

nc pc2-student-b 8080

Type a message and press Enter. Your partner hears it arrive. This confirms that hostname resolution, routing, and the service are all working.

### Discussion Questions — Scenario 2

- Why will nslookup always fail for local lab names when the DNS server is 8.8.8.8?

- What is the hosts file and at what point in the name resolution process does Windows consult it?

- Why do you need to run ipconfig /flushdns after editing the hosts file?

- What does it mean when nslookup returns the correct IP but the connection to port 8080 still fails?

Scenario 3: The Ghost on the Network

Fault report: Connectivity to 172.16.100.11 is intermittent. Sometimes ping works, sometimes it times out, and nothing has changed on either machine. Your instructor suspects an IP address conflict.

### Your Task

An IP conflict means two machines share the same IP address. ARP will reveal this because the same IP will appear with two different MAC addresses at different times.

- Ping your partner to create a fresh ARP entry.

ping 172.16.100.11

- Immediately check the ARP cache.

arp -a

A healthy entry sounds like:

172.16.100.11     aa-bb-cc-dd-ee-ff     dynamic

In a conflict, either the same IP appears twice with different MACs, or the MAC address changes each time you run arp -a because two devices are racing to answer ARP requests.

- Clear the cache and repeat the sequence two or three times, listening for the MAC address to change.

arp -d *

ping 172.16.100.11

arp -a

If the MAC shown for 172.16.100.11 changes between runs, there is a confirmed IP conflict on the subnet.

- Identify the intruder. Run arp -a and compare every MAC in the table against the known MACs of your two lab machines (found earlier with ipconfig /all). Any unrecognized MAC belongs to the conflicting device.

arp -a

- Fix the conflict by giving the conflicting device a different IP. Once resolved, clear the cache and retest.

arp -d *

ping 172.16.100.11

### Discussion Questions — Scenario 3

- Why does intermittent connectivity point toward an IP conflict rather than a faulty cable?

- At what layer of the TCP/IP model does ARP operate, and why is it specific to the local subnet?

- What would happen on a large production network if two servers shared the same IP address?

Scenario 4: Renew Fails After a Power Cut

Fault report: After a power cut, your machine came back with an address starting with 169.254 instead of 172.16.100.x. This is called an APIPA address and it means DHCP failed. You need to investigate and recover.

### Your Task

- Confirm the APIPA address.

ipconfig

You will hear an address beginning with 169.254. Windows sent DHCP Discover broadcasts and received no response, so it assigned itself a link-local address. You cannot reach any host on 172.16.100.x while your address is in the 169.254 range.

- Get full detail to understand the DHCP state.

ipconfig /all

Listen for: DHCP Enabled says Yes, Autoconfiguration Enabled says Yes, and DNS Servers shows 8.8.8.8. These confirm that DHCP is turned on but the server did not respond. You will also hear the APIPA address reported as Autoconfiguration IPv4 Address.

- Attempt to contact the gateway. This will likely fail, but the result is informative.

ping 172.16.100.1

Expected: Request timed out. This confirms your address is the problem, not the gateway itself. An APIPA address in the 169.254 range cannot route to 172.16.100.x.

- Force a new DHCP lease.

ipconfig /release

ipconfig /renew

If the DHCP server is back online, you will hear your 172.16.100.10 address returned along with the mask, gateway, and DNS server 8.8.8.8. If you hear Unable to contact your DHCP server, the server is still unreachable and needs to be restarted.

- Verify the address is restored.

ipconfig

ping 172.16.100.11

### Discussion Questions — Scenario 4

- What is the DORA process and which specific step fails when you end up with an APIPA address?

- Why can two APIPA machines sometimes communicate with each other but not with the rest of the network?

- What is the difference between ipconfig /release and ipconfig /renew? Do you always need to release before renewing?

Scenario 5: The Port That Was Not Listening

Fault report: You are trying to connect to a service on 172.16.100.11 port 5000. Your partner says they started it, but you receive Connection refused every time. Both machines can ping each other.

### Your Task

Since ping works, Layer 3 is fine. The fault is at the transport layer. Verify whether the service is actually listening before assuming anything else is wrong.

- Attempt the connection and listen carefully to the exact error message.

nc 172.16.100.11 5000

You should hear: connect to 172.16.100.11 port 5000 failed: Connection refused.

Connection refused means the host is alive and reachable but nothing is bound to port 5000. This is different from Request timed out, which would mean the host itself is unreachable.

- Ask your partner to run netstat on their machine and listen for port 5000.

netstat -an

A working listener sounds like:

TCP    0.0.0.0:5000    0.0.0.0:0    LISTENING

If port 5000 is absent from the output, the service is not running. That is the root cause.

- Your partner starts the listener.

nc -l -p 5000

- Your partner opens a second terminal and confirms port 5000 is now listed.

netstat -an

- You try the connection again.

nc 172.16.100.11 5000

Type a message and press Enter. Your partner hears it arrive. The session is now ESTABLISHED.

- While the session is open, both of you run netstat and listen for the ESTABLISHED state.

netstat -an

You should each hear a line showing your IP addresses and port 5000 with the word ESTABLISHED. One line appears on each machine, confirming the TCP three-way handshake completed successfully.

### Discussion Questions — Scenario 5

- What is the difference between Connection refused and Request timed out? What does each error tell you about where the fault is?

- Why does netstat show ESTABLISHED on both machines simultaneously during an active TCP session?

- If netstat showed port 5000 in LISTENING state but the connection still failed, what would you check next?

End of Student Lab